ESP32 Web Dashboard

## Project Overview

This project demonstrates a basic IoT dashboard served by an ESP32.
It displays live sensor data (temperature and humidity from a DHT11/DHT22 sensor) and allows toggling a digital output (LED or relay) via a web interface.

## Hardware Requirements

| Component | Connection / Notes |
| --- | --- |
| ESP32 | Any standard ESP32 development board |
| DHT11 / DHT22 | DATA → GPIO 4, VCC → 3.3V, GND → GND |
| LED / Relay | Positive → GPIO 2 (with resistor if LED), Negative → GND |

## Software Requirements

- Arduino IDE or PlatformIO

- ESP32 board support installed

- Libraries:

  - WiFi.h

  - WebServer.h

  - DHT.h (Adafruit DHT Sensor Library)

## Features

- Live temperature and humidity readings on a web dashboard

- Toggle LED or relay from the dashboard

- Updates every 2 seconds without refreshing the page

- Serial Monitor displays live sensor readings and LED state

## Usage Instructions

- Connect the hardware as per the table above.

- Open the sketch in Arduino IDE.

- Modify WiFi credentials:

const char* ssid = "YOUR_WIFI_NAME";

const char* password = "YOUR_WIFI_PASS";

- Upload the sketch to the ESP32.

- Open Serial Monitor (115200 baud) to see IP address and live readings.

- Open a web browser and navigate to:

http://<ESP32_IP_ADDRESS>

- Use the dashboard:

  - Temperature and Humidity are updated every 2 seconds.

  - Click Toggle to turn the LED/relay ON or OFF.

## Dashboard Layout

- Temperature Card – Shows live temperature

- Humidity Card – Shows live humidity

- LED Control Card – Toggle LED/relay, shows current state

## Demo Notes

- Serial output example:

Temperature: 24.5 °C, Humidity: 55 %, LED: OFF

LED Toggled: ON

Temperature: 24.6 °C, Humidity: 55 %, LED: ON

- The dashboard works on desktops, tablets, and smartphones.

Arduino Code

#include <WiFi.h>

#include <WebServer.h>

#include "DHT.h"

// ================= WIFI =================

const char* ssid = "Wokwi-GUEST";   // Change to your WiFi SSID

const char* password = "";           // Change to your WiFi password

// ================= DHT =================

#define DHTPIN 4

#define DHTTYPE DHT22

DHT dht(DHTPIN, DHTTYPE);

// ================= OUTPUT =================

#define LED_PIN 2

bool ledState = false;

WebServer server(80);

// ================= HTML DASHBOARD =================

String webpage = R"rawliteral(

<!DOCTYPE html>

<html>

<head>

<meta name="viewport" content="width=device-width, initial-scale=1">

<title>ESP32 IoT Dashboard</title>

<style>

body { font-family: Arial; background:#f0f2f5; text-align:center; }

.container { max-width:400px; margin:auto; }

.card {

background:white;

padding:20px;

margin:15px;

border-radius:10px;

box-shadow:0 4px 8px rgba(0,0,0,0.2);

}

button {

padding:12px 20px;

font-size:16px;

border:none;

border-radius:6px;

cursor:pointer;

}

.on { background:green; color:white; }

.off { background:red; color:white; }

</style>

</head>

<body>

<h2>ESP32 Web Dashboard</h2>

<div class="container">

<div class="card">

<h3>Temperature</h3>

<p id="temp">-- °C</p>

</div>

<div class="card">

<h3>Humidity</h3>

<p id="hum">-- %</p>

</div>

<div class="card">

<h3>LED Control</h3>

<button id="btn" onclick="toggleLED()">Toggle</button>

</div>

</div>

<script>

function updateData(){

fetch("/data")

.then(response => response.json())

.then(data => {

document.getElementById("temp").innerHTML = data.temp + " °C";

document.getElementById("hum").innerHTML = data.hum + " %";

let btn = document.getElementById("btn");

if(data.led == 1){

btn.className="on";

btn.innerHTML="ON";

} else {

btn.className="off";

btn.innerHTML="OFF";

}

});

}

function toggleLED(){

fetch("/toggle");

}

setInterval(updateData,2000);

updateData();

</script>

</body>

</html>

)rawliteral";

// ================= HANDLERS =================

void handleRoot() {

server.send(200, "text/html", webpage);

}

void handleData() {

float t = dht.readTemperature();

float h = dht.readHumidity();

// Print to Serial Monitor

Serial.print("Temperature: ");

Serial.print(t);

Serial.print(" °C, Humidity: ");

Serial.print(h);

Serial.print(" %, LED: ");

Serial.println(ledState ? "ON" : "OFF");

String json = "{";

json += "\"temp\":" + String(t) + ",";

json += "\"hum\":" + String(h) + ",";

json += "\"led\":" + String(ledState ?  0  :  1);

json += "}";

server.send(200, "application/json", json);

}

void handleToggle() {

ledState = !ledState;

digitalWrite(LED_PIN, ledState);

Serial.print("LED Toggled: ");

Serial.println(ledState ? "Off" : "On");

server.send(200, "text/plain", "OK");

}

// ================= SETUP =================

void setup() {

Serial.begin(115200);

dht.begin();

pinMode(LED_PIN, OUTPUT);

digitalWrite(LED_PIN, LOW);

Serial.println("Connecting to WiFi...");

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println("\nConnected!");

Serial.print("ESP32 IP Address: ");

Serial.println(WiFi.localIP());

Serial.println("Dashboard is ready!");

server.on("/", handleRoot);

server.on("/data", handleData);

server.on("/toggle", handleToggle);

server.begin();

}

// ================= LOOP =================

void loop() {

server.handleClient();

// Print sensor & LED every 2 seconds

static unsigned long lastPrint = 0;

if (millis() - lastPrint > 2000) {

lastPrint = millis();

float t = dht.readTemperature();

float h = dht.readHumidity();

Serial.print("Temperature: ");

Serial.print(t);

Serial.print(" °C, Humidity: ");

Serial.print(h);

Serial.print(" %, LED: ");

Serial.println(ledState ? "ON" : "OFF");

}

}